# USB to Serial Device Driver

# For Windows CE .NET

MosChip Semiconductor Technology Ltd

Version 0.1, November 09, 2004

- Contents

1. Introduction	1

2. Resource Requirements	1

3. Install the Driver	1

4. Test the device with the direct cable connection	2

## 1. Introduction

- This document provides the information about how to install the driver for MOSCHIP MCS 7720 USB2Serial device on windows CE .NET 4.2 environments using the CAB file.

## 2. Resource Requirements

- For installation of the driver using .cab file, the following resources are needed:

- CEPC/Windows CE Embedded Device with Windows CE 4.2 OS

- PC having Win 2k with Microsoft ActiveSync.

- NULL Modem Cable (internally connected strictly according to RS232 standard)

- MCS 7720 USB2Serial device.

## 3. Install the Driver

The Installation of the MCS7720 Driver by a .cab file can be done through the following steps:

- In the PC, Microsoft ActiveSync should be installed.

- Connect the WinCE Device (CEPC) and the PC through a Null modem cable.

- Run ActiveSync in the PC.

- Open the Command Prompt in WinCE Device and type the command ‘repllog’ and press Enter.

- A New Partnership window appears in ActiveSync. Select No and then click on Next.

- Then it will establish a connection between PC and WinCe Device and it will show the connection status as Connected on ActiveSync.

- Then click on the Explore button on the ActiveSync window, and copy the User20Ce WinZip (it is actually a CAB file) file to the Mobile Device folder on your PC.

- Then open My Computer in the WinCe Device (CEPC) and the CAB file (User20Ce) will be there.

- To install the driver, just double click on the CAB file (User20Ce).

- An installation window will appear. Click OK to install. The driver is now installed.

- Now, insert the MCS 7720 USB2Serial device in to USB port on the WinCE Device. The driver will be loaded and the device is ready for connection to other machines.

## 4. Test the device with the direct cable connection

- After plugging the device on the USB port, click on the Start button.

- Go to Settings and click on the Network and Dial-up Connections

- Double click on the Make New Connection.

- Select Direct Connection and Press Next.

- Select the Device Usb Serial Cable on COMx ( Where x is the port number )

- Press TCP/IP Settings.

- Uncheck the Use server-assigned IP address, Use Software Compression and Use IP header compression. And give the IP address (e.g. 192.168.1.1) and click on OK.

- Connect one of the MCS7720 serial port to standard serial port of the other machine using the Null Modem cable. Configure the other machine serial port for accepting the incoming connection.

- Double click the My Connection to establish the direct cable connection.

- Dialog box appears which tells the connection status/progress.

- Message box displays “Connected” when the communication is established.

- Then Run the Ping command to test the hardware.